Unisoft Water Softener:

HOW DOES A WATER SOFTENER WORK?

"How does a water softener work?" is a common question.  Hard water is water that has a high mineral content, typically calcium or magnesium. When it enters your home from your water source, it can leave deposits or “scale” on plumbing fixtures, appliances, and other everyday objects. It can clog pipes and reduce the efficiency of appliances. It affects your everyday life –from cooking to cleaning to doing laundry.

A water softener has a resin tank containing thousands of tiny resin beads that hold electrically charged ions. When freshly regenerated, the beads become full of sodium ions from the water softener salt. As hard water passes through the water softener system, the calcium and magnesium ions are attracted to the resin beads. There, sodium ions are exchanged for the hard water ions. The softener system then delivers soft water for your use.

## OUR WATER SOFTENING PLANT'S SALIENT FEATURES:

• Corrosion resistant resin and brine tank construction.
• Flow-based or Timer-based automatic regeneration.
• High quality softener resin provides stability and uniform size for top performance and   long life.
• Simple inline configuration for ease of installation and maintenance.
• Range of softener systems to suit Institutional, commercial and industrial applications..
• Softeners are available in FRP, MS, SS, Operating in automatic mode.

OUR WATER SOFTENING PLANT'S BENEFITS:

• Easy to install and operate.
• Produces clear, soft and non – scale forming water.
• Low operating costs.
• Incorporates high-capacity polystyrene bead type cation exchange resin which is remarkably stable and has a long life.
• Total Hardness removal.

Unisoft Duel Media Filter (PSF & ACF):

HOW DOES A Duel media WORK?

Dual media filter contains anthracite / black carbon along with fine sand reinforced by marbled and gravels. These filters consist of a coating of anthracite (1.25-2.5mm) resting on a coating of fine sand (1-1.5mm) Anthracite is coarse and has more dirt holding capacity as compared to fine sand. Dual media filters are used in places where the raw water contains suspended contaminants, turbidity and iron. Dual media filters provide very efficient particle removal under the conditions of high filtering system amount. These filters use layers of anthracite and garnet contaminants as the filter medium, with filtering system amount of 98% down to 2 microns.

## OUR DUAL MEDIA FILTER'S ADVANTAGES:

- Fully Automatic operation, flow, time/day basis.

- Efficient Turbidity and TSS Removal.

- Filter up to 20 – 30 Microns.

- FRP, MSEP, SS Vessel available.

- 4 Stage Multi-layer filters.

- Low Pressure drops across the vessel.

## OUR DUAL MEDIA FILTER'S SALIENT FEATURES:

• Dual media filters have high dirt removal capacity of the finest sand forms.
• Dual media filters are five times more effective in removing dirt than the pressure filters.
• Dual media filters provide very efficient particle removal under the conditions of high filtration rate.
• Inside a sand-anthracite filter is a layered bed of filter media.
• Sand is used to remove the suspended particles and anthracite is used to remove the odor and color etc. to make the water fit for different applications.

Unisoft Multi-functional Flow Control Valve (Automatic Malty Port Valve) Specification:

Certification(s): CE, ROHS

Technical Parameter:

Controller model: Volume type / Down-flow

Suited pressure: 0.1 - 0.6MPa

Suited water temperature: 5-45℃

Water Capacity: 4.5t/h

Tank Diameter Range for Softens: 6”- 42”

Water Inlet/Outlet: 1” to 3’’ (Outer tooth)

Drain: 1/2” to ¾’’ (Outer tooth)

Brine Tank Connection: 3/8”M (Outer tooth)

Rinse Pipe: 1.05”OD (26.7mm)

Base (The connector for tank): 2-1/2” 8NPSM

Unisoft Fully Automatic Malty Port Valve Features:

1.  More reliable way of opening and close

2.  Manual function:  When it is required, will operate by manual keys.

3.  Keyboard locked function: The key set will lock automatic exceed one minute, you must open the lock over again when you operate, prevent wrong operation.

4.  It adopts LED dynamic colorful screen: The colorful screen flicker express it is in the state of running, otherwise express it is in the state of regeneration.

5.  Prompt power cut secularly: Out of electricity exceed three days, you must reinstall the currently time when the timepiece flicker ceaselessly, the already setting time is not changed.

6.  No coming water while single pot type control valve regeneration

7.  Having output signal connector: via three port of open, close, common down-lead to connect the electromagnetism valve or increase-pressure pumps.

8.  Two modes of flow meter: By changing the process parameter to realize to change the mode to be volume delayed (A-01) or volume timely (A-02).

9.  It could set up maximum interval regeneration days: This parameter means no matter how much flux it is set, or how much flux treated, when it exceeds the maximum interval generation days (could be set within 0 to 40 days), than it will start up regeneration although the flux is not reached the set value.

Unisoft Water Softener Advantages:

### Healthier, Softer Skin

### Shiny, Strong Hair

### Clothes Are Softer and Look Cleaner

### Removes Any Unwanted Tastes from Your Drinking Water

### Soft Water Is Easier on Your Pipes and Appliances

### Softer Water Will Save You Money

Unisoft Water Softener Application:

- Residential, Apartments.

- Commercial establishment.

- Hotels & Restaurants.

- Schools and Hostels.

- Hospitals.

- Factories, Garments,

- Pools, etc….